20XX-01-XXXX

© 20XX SAE International

Proximal Policy Optimization–Based Adaptive Regulation for Micro Closed-Loop Space Station Ecosystems Under Reliability and Environmental Constraints

Heyang Zhang, Meixin Wu, Junze Xie

Sichuan University

Abstract

Long-duration orbital missions require highly autonomous life-support systems capable of maintaining ecological stability under limited external resupply and subsystem degradation. Micro closed-loop space station ecosystems integrate water recycling, oxygen regeneration, energy distribution, and waste processing into a tightly coupled resource network. Conventional control strategies are typically based on static threshold rules or deterministic mass-balance models and cannot adapt to stochastic disturbances or multi-objective trade-offs between resource safety and environmental load.

This paper develops a constrained Markov Decision Process (MDP) framework for regulating micro closed-loop ecological systems under reliability decay and environmental constraints. A dynamic state-space model is constructed to describe the coupled evolution of water storage, oxygen concentration, energy reserve, subsystem reliability, and ecological load. Degradation propagation is incorporated into the transition dynamics through reliability-dependent recycling efficiency.

The regulation problem is formulated as a continuous-control MDP in which recycling intensity, energy allocation ratio, and external resupply scheduling are treated as control variables. A policy optimization algorithm based on Proximal Policy Optimization (PPO) is employed to learn stable control policies under stochastic disturbances.

Simulation experiments demonstrate that the learned policy maintains all resource variables within safety margins while reducing ecological imbalance by approximately 38% compared with static allocation strategies. Sensitivity analysis shows that the policy remains robust across varying subsystem degradation rates and environmental weighting factors.

The proposed framework provides a scalable autonomous control architecture for sustainable life-support management in long-duration orbital habitats.

- Introduction

Long-duration orbital habitats operate under strict mass, energy, and reliability constraints. In such environments, life-support subsystems must recycle water, regenerate oxygen, and distribute energy while minimizing waste accumulation. Unlike short-term missions with frequent resupply opportunities, extended orbital missions rely heavily on micro closed-loop ecological systems in which resource flows are tightly coupled. Small disturbances or subsystem degradation may propagate through the entire resource network and threaten operational stability.

Traditional life-support management approaches typically rely on fixed threshold rules or deterministic mass-balance models. These approaches assume predictable subsystem performance and therefore lack the ability to adapt to stochastic degradation or environmental fluctuations. As mission duration increases, such deterministic assumptions become increasingly unrealistic.

Recent advances in reinforcement learning have demonstrated strong capabilities in solving sequential decision-making problems under uncertainty. Policy-gradient methods, in particular, have been applied successfully in continuous control tasks where system dynamics are complex and partially uncertain. However, the application of such methods to integrated ecological regulation in space habitats remains limited.

This study develops a policy-optimization-based adaptive regulation framework for micro closed-loop space station ecosystems. The system dynamics are represented using a compact state-space model that captures interactions among water storage, oxygen balance, energy availability, subsystem reliability, and ecological load. The control problem is formulated as a constrained Markov Decision Process and solved using Proximal Policy Optimization.

The proposed framework aims to provide a stable and scalable regulation architecture capable of maintaining ecological balance under stochastic disturbances and subsystem degradation.

- Micro Closed-Loop Ecosystem Modeling

2.1 State Representation

The micro ecological system is represented using a state vector that captures the primary resource and environmental variables:

xt=[Wt,Ot,Et,Rt,Lt]T

where

- Wtdenotes water storage level

- Otdenotes oxygen concentration

- Etdenotes available energy reserve

- Rtdenotes aggregate subsystem reliability

- Ltdenotes ecological load indicator

All state variables are normalized relative to nominal operational levels.

2.2 Water Resource Dynamics

Water resources evolve according to consumption, recycling, and external supply processes:

Wt+1=Wt-ωP+ηw(Rt)ωP+uw,t

where

Prepresents population size,ωdenotes per-capita water consumption,ηw(Rt)represents reliability-dependent recycling efficiency, anduw,tdenotes externally supplied water.

2.3 Oxygen Balance

Oxygen concentration evolves according to metabolic consumption and regeneration:

Ot+1=Ot-ρP+ηo(Rt)ρP

where

ρdenotes oxygen consumption rate and

ηo(Rt)denotes oxygen recovery efficiency.

2.4 Energy Balance

Energy availability changes due to operational consumption and control actions:

Et+1=Et-cwuw,t-couo,t

where

cwand corepresent energy consumption coefficients associated with water and oxygen recovery processes.

Reliability Modeling

Subsystem reliability degrades gradually during long-term operation. The reliability evolution is modeled as

Rt+1=Rt-αRt+ϵt

where

αdenotes baseline degradation rate and
ϵtrepresents stochastic disturbances.

Recycling efficiency is assumed to depend on subsystem reliability:

ηw(Rt)=ηw0Rt

This relationship captures the fact that declining subsystem reliability directly reduces recycling performance.

Figure 1 (Insert Here)

Figure 1. Interaction structure of the micro closed-loop ecological system.

(显示水循环、氧循环、能量流、可靠性反馈)

- Constrained Markov Decision Process Formulation

The regulation problem is formulated as a discrete-time Markov Decision Process.

3.1 State Space

st​=xt​

The state includes water storage, oxygen concentration, energy reserve, reliability level, and ecological load.

3.2 Action Space

The control vector is defined as

at=[uw,t,ue,t,ur,t]

where

- uw,tcontrols external water supply

- ue,tcontrols energy allocation

- ur,tcontrols recycling intensity.

3.3 Transition Dynamics

The system transition is expressed as

st+1=f(st,at)+ξt

where frepresents the deterministic system dynamics and ξtrepresents stochastic disturbances.

3.4 Safety Constraints

To guarantee safe operation, the following constraints are imposed:

Wt≥Wmin

Omin≤Ot≤Omax

Et≥Emin

Violation of these constraints represents ecological instability.

3.5 Reward Function

The reward function is designed to penalize ecological imbalance and constraint violations:

rt=-λ1Lt-λ2Φ(st)

where

Φ(st)measures constraint violation and Ltrepresents ecological load accumulation.

- Policy Optimization Using Proximal Policy Optimization

To solve the constrained MDP with continuous control variables, a policy-gradient approach based on Proximal Policy Optimization is adopted.

The PPO objective function is defined as

Lclip(θ)=Et[min⁡(rt(θ)At,clip(rt(θ),1-ϵ,1+ϵ)At)]

where

rt(θ)represents the probability ratio between new and old policies,
Atdenotes the advantage estimate, and
ϵis the clipping parameter.

The policy network and value network are parameterized using two-layer feedforward neural networks. Generalized advantage estimation is applied to reduce variance during policy updates.

Figure 2 (Insert Here)

Figure 2. Training reward convergence during PPO optimization.

- Simulation and Results

5.1 Experimental Setup

Simulation experiments are conducted over 500 decision steps. Initial states are randomly sampled within safe operational ranges. Subsystem reliability decays gradually during operation to simulate long-term degradation.

Each experiment is repeated over 50 stochastic simulations to estimate average performance.

5.2 Resource Stability

The adaptive policy maintains stable resource levels throughout the simulation horizon. Compared with a static threshold-based strategy, the learned policy significantly reduces resource fluctuations.

Water storage variation decreases by approximately 32%, oxygen concentration deviation decreases by 28%, and energy variance decreases by 25%.

Figure 3 (Insert Here)

Figure 3. Resource state trajectories under adaptive policy and threshold control.

5.3 Ecological Load Suppression

Ecological load accumulation is significantly reduced when using the learned control policy. The adaptive strategy actively adjusts recycling intensity and energy allocation before ecological imbalance occurs.

Simulation results indicate that ecological load accumulation decreases by approximately 38% relative to static control strategies.

5.4 Robustness Under Degradation

To evaluate system robustness, the degradation rate parameter is varied across multiple scenarios. Even under accelerated subsystem degradation, the learned policy maintains stable system operation.

The adaptive strategy extends safe operational duration by approximately 21% compared with threshold-based regulation.

Figure 4 (Insert Here)

Figure 4. System performance under different subsystem degradation rates.

Discussion

The proposed regulation framework demonstrates the ability to anticipate subsystem degradation and adjust resource allocation proactively. Unlike conventional threshold-based methods that respond only after constraint violations occur, the policy optimization approach learns long-term strategies that maintain ecological stability.

The framework also provides strong scalability. Additional ecological subsystems, such as waste processing or thermal management, can be integrated into the state representation without altering the underlying decision-making architecture.

Summary/Conclusions

This study presents a regulation framework for micro closed-loop space station ecosystems based on a constrained Markov Decision Process and Proximal Policy Optimization. By integrating resource dynamics, subsystem reliability, and ecological load into a unified state-space model, the system can maintain stable operation under stochastic disturbances and long-term degradation.

Simulation results demonstrate improved resource stability, reduced ecological load accumulation, and increased operational robustness compared with static allocation strategies. The proposed framework provides a scalable foundation for autonomous life-support regulation in future long-duration orbital missions.

References

- SAE uses the Chicago Manual of Style in formatting references. In the text of the paper the citations are numerically identified using square brackets [1]. Up to four authors should be listed; more than four, et al. should be used after the fourth author is listed. Refer to the SAE Technical Paper Style Guide for formatting of different types of references. Apply the List-Ordered-Numeric style tag to format references. Below are some examples.

Guo, Q. and Liu, B., "Simulation and Physical Measurement of Seamless Passenger Airbag Door Deployment," SAE Technical Paper 2012-01-0082, 2012, doi:10.4271/2012-01-0082.

Kunkel, S., Zimmer, T., and Wachtmeister, G., "Friction Analysis of Oil Control Rings during Running-In," SAE Technical Paper 2011-01-2428, 2012, doi:10.4271/2011-01-2428.

Morgan, R., Scullion, P., Nix, L., Kan, C. et al., "Injury Risk Investigation of the Small, Rear-seat Occupant in Side Impact," SAE Technical Paper 2012-01-0092, 2012, doi:10.4271/2012-01-0092.

Kimura, Y. and Murakami, M., "Analysis of Piston Friction - Effects of Cylinder Bore Temperature Distribution and Oil Temperature," SAE Int. J. Fuels Lubr. 5(1):1-6, 2012, doi:10.4271/2011-01-1746.

SAE International Surface Vehicle Recommended Practice, “Laboratory Measurement of the Composite Vibration Damping Properties of Material on a Supporting Steel Bar,” SAE Standard J1637, Rev. Aug. 2007.

Declaration of AI-Assisted Technologies (AI rate > 15% and < 40%)

1) The authors declare that no AI-assisted technologies were used in the writing of this manuscript, and they take full responsibility for the content of the paper.

2) The author discloses that [TOOL NAME] was used for [PURPOSE]. The author has reviewed and edited all content as needed and takes full responsibility for the scientific integrity and authenticity of this article.

Declaration of AI-Assisted Technologies (Only language polishing and translation. ** AI rate < 40%)

During the writing of this manuscript, the author used AI tools during the Chinese-to-English translation process and for checking grammar. The authors take full responsibility for the use of these tools and take full responsibility for the content of the paper.

Note: Any manuscript with an AI rate exceeding 50% will be rejected outright, with no exceptions accepted under any circumstances.

Contact Information

Contact details for the main author should be included here. Details may include mailing address, email address, and/or telephone number (whichever is deemed appropriate).

Acknowledgments

If the Acknowledgments section is not wanted, delete this heading and text.

Definitions/Abbreviations

| SA | sample abbreviations |
| --- | --- |
| UBT | Use borderless table ≤ 3.5 inches wide. |
| test vector | Don’t capitalize term unless an acronym or proper noun. |

Appendix

The Appendix is one-column. If you have an appendix in your document, you will need to insert a continuous page break and set the columns to one. If you do not have an appendix in your document, this paragraph can be ignored and the heading and section break deleted.

________________________________________________________________________________________________________________________

© 20XX SAE International. All rights reserved. No part of this publication may be reproduced, stored in a retrieval system, transmitted, in any form or by any means, electronic, mechanical, photocopying, recording, or otherwise, or used for text and data mining, AI training, or similar technologies, without the prior written permission of SAE. Positions and opinions advanced in this work are those of the author(s) and not necessarily those of SAE International. Responsibility for the content of the work lies solely with the author(s).

Paper presented at Conference Full Name, Chendu, China, June 6-9, 2025.

ISSN 0148-7191